README.DOC for \Driver\LPTI

===========================

There are six files and one sub-directory (lpsource) in this directory.

lpsource subdirectory

=====================

The lpsource subdirectory contains the following files:

--AIX.txt

--DECOSF1.txt

--HP.txt

--SCO.txt

--Sun4.txt

--Sun5.txt

--SVR4.txt

--unixware.txt

These text files contain full instructions for installing and using

the PSfilter printing system, for each of the supported systems.

Please read the appropriate file for your system.

--psfilter.c, errormsg.h

psfilter.c & errormsg.h are the source files used to generate the

PSfilter executable program.

--PS.aix41ppc, PS.sco3, PS.sco5, PS.sun5

These four executable files are provided in case there is no C Compiler

in your AIX 4.1 Power PC, SCO 3.x, SCO 5.x or SUN 5.x UNIX system.

To use any of these four programs, you need to copy the program file to

a new file called "PSfilter" in the LPsource directory.

Files in this Directory

=======================

The files in this directory are:

--readme.doc

This file.

--doc.ps

This is a PostScript data file containing 04 ASCII codes at its

beginning and end.  It is used for testing a PostScript printer.

--dumb_int.sh

dumb_int.sh is a Bourne Shell script.  It is an interface program of

a dumb printer for System V Unix.  It pipes the banner page and data

files to the PSfilter program.

--install.sh

install.sh is provided to install the Psfilter printing system on the

UNIX host

.install.sh attaches one printer at a time; run it again to install

additional printers.

--instmsgs

This file contains all output messages of install.sh

--mycc

The mycc program is used to compile the psfilter.c source code.

Its output execution file is PSfilter in the subdirectory ./lpsource.

mycc will be invoked by install.sh

***************** Overview of Installation Procedure *****************

1. Install the PrintMate in your LAN, and assign a valid IP Address,

Netmask, and Gateway.

2. Navigate to the destination directory on your system.

Insert the CD-ROM into your CD-ROM device and type the command:

tar xvf CD-ROM mounted directory/driver/lptitar/lpti.tar or

execute CD-ROM mounted directory/driver/lpti/install.sh directly

3. Change to the ./LPTI directory created by the previous command and

execute install.sh. (NOT on AIX! Refer to AIX.txt for details.)

cd LPTI

./install.sh

(Refer to the .txt file for your operating system for full details

on using install.sh)

4. On system V, print files by the following command:

lp -d printer_name  file_names

On BSD system , print files by the following command:

lpr -P printer_name  file_names

(Refer to the .txt file for your operating system for details of

the parameters which can be used with Psfilter when it is invoked

within the interface script.)