LANMANAGER REMOTE PRINT SERVICE   (Version 3.x NetBeui only)

------------------------------------------------------------

This version of the driver software is only compatible with

Firmware version 03.03 upwards.

INSTALLING THE SOFTWARE

-----------------------

1.  Place the HBM Remote print service distribution disk in drive A

of your server.

2.  Open an OS/2 Full Screen Command Session and create a directory

for Remote Print Services on your hard disk:

MD C:\HBMREM

3.  Copy all the files to the new directory from the distribution disk

in drive A.

COPY A:\*.* C:\HBMREM

SOFTWARE COMPONENTS

-------------------

The following is a list of files you should have in the C:\HBMREM directory:

ADDPORT.CMD  -  To add ports on the server

DELPORT.CMD  -  To remove unused ports from the server

PORTCHG.EXE  -  Program used by the above commands

REMPRT3.EXE  -  Remote Print Service program

REMBEU.EXE   -  Remote Print Service driver

REMSTAT3.EXE -  Printer status program

CONFIGURING NEW PRINTERS

------------------------

This section explains how to configure new printers on a LAN Manager Server

for use with the remote print service.

You will need administration rights on the server to configure printers

and queues.

At your server, using an OS/2 Full screen command session proceed as follows:

1.  Change to the Remote Print Service directory.

CD \HBMREM

2.  Create Ports for your new printers using the ADDPORT command, choose

a name that will help you identify the printer, maximum length 8

characters.     e.g. HPLASER, SALESPR

ADDPORT HPLASER

ADDPORT SALESPR

...

3.  Follow the OS/2 procedure for installing printers and creating queues

attached to the server. You must install and select either the

IBMNULL or MSNULL printer driver. For the DEVICE to connect to choose

a port name added in step 2 above.  e.g. HPLASER.

4.  In the Remote Print Service directory 'HBMREM' create an initialisation

file called REMPRT.INI. This file is used by remote print service at

startup time and assigns the remote printer names to the device port

names. The format of the file is as follows:

<Printer name> <device port> </P> </Q> </H>

Printer name    This is the remote printer interface Printserver

name. usually 'NETPRINTERS'

Device port     This is the name selected for the DEVICE to

connect to when installing the printer.

/P              Permanent Retry on printer connection. (Default)

This switch enables continuous retries to be

performed if the printer has been switched off.

If not specified a warning message will be

produced after 60 secs with the option to cancel

the print, abort the service or retry the print.

/N              Notify after 60 secs it printer not connected.

/Q              Quiet Mode.

This switch disables the beep warning when the

printer is busy.

/H              Hide Errors.

This switch disables the popup error windows

e.g  NETPRINTERS HPLASER

RP21 SALESPR /P

....

...

5.  Edit your STARTUP.CMD and add the following line at the end:

\HBMREM\REMPRT3 @REMPRT.INI

This will ensure that the remote print service is initialised each time

you start the server. If you wish to manually start the print service,

you can execute the REMPRT3.EXE file from the OS/2 command prompt.

6.  Using the LAN Manager Administration facility (NET ADMIN) add the

newly created printers to your list of shared resources.